|  | PRESS RELEASE |
| --- | --- |

In May, one fuel sample tested failed

(Prague, 31 July 2024) The Czech Trade Inspection Authority monitored the quality of fuels in May 2024. It took a total of 180 samples at filling stations in the Czech Republic and one sample failed to meet the specified quality requirements.

This represented 0.6% of non-compliant samples within the overall assessment. It was one diesel fuel sample.

| Collected fuel samples by types - May 2024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel type | collected samples | in % | noncompliant samples | % of fuel type |
| petrol | 75 | 41.6 | 0 | 0,0 |
| diesel fuel | 84 | 46.7 | 1 | 1,2 |
| FAME | 0 | 0.0 | 0 | 0,0 |
| diesel fuel blebd | 0 | 0.0 | 0 | 0,0 |
| diesel fuel B10 | 0 | 0.0 | 0 | 0,0 |
| high FAME diesel fuel | 0 | 0.0 | 0 | 0,0 |
| paraffinic diesel fuel | 0 | 0.0 | 0 | 0,0 |
| LPG | 18 | 10.0 | 0 | 0,0 |
| CNG, bio-CNG | 3 | 1.7 | 0 | 0,0 |
| LNG, bio-LNG | 0 | 0.0 | 0 | 0,0 |
| Ethanol E85 | 0 | 0.0 | 0 | 0,0 |
| Total | 180 | 100.0 | 1 | 0,6 |

Diesel fuel

In May, the CTIA collected and inspected a total of 84 diesel fuel samples. This number of samples taken and inspected included one sample that failed the quality requirements of the applicable technical standard. The inspected diesel fuel sample failed the quality parameter 'pre-distilled volume at 250 °C', where the measured value of this quality parameter was 74.0 % v/v (the maximum value including measurement uncertainty is 66.5 % v/v) and in the quality parameter 'density at 15 °C', where the measured value of this quality parameter was 813.6 kg/m3 (the minimum value including measurement uncertainty is 819,7 kg/m3 and the maximum value including measurement uncertainty is 845.3 kg/m3).

The other samples of diesel fuel inspected and analysed met the quality parameters set out in the technical standard EN 590/O1:2023 - Motor fuels - Diesel fuels - Technical Requirements and Test Methods in relation to the requirements of Decree No 516/2020 Coll., on Requirements for Fuel and Implementation of Certain Other Provisions of the Act on Fuels.

Biofuels in fuels

During the monitored period, the content of fatty acid methyl esters was inspected in 84 samples diesel fuel. The ethanol content in automotive petrol was inspected in 75 samples. The laboratory analyses did not show that the permissible upper limit for the content of organic compounds was exceeded in the samples of motor fuels monitored.

Conclusion

In May, as in April, 1 sample of diesel fuel collected and inspected failed. Compared to April, there was an improvement in May from 1.0 % to 0.0 % for the samples of motor gasoline taken and tested and from 2.7 % to 0.0 % for the samples of LPG taken and inspected.

Due to fuel market developments, samples of FAME, diesel fuel blend, diesel B10, high FAME diesel, paraffinic diesel, bio-CNG, LNG, bio-LNG and E85 Ethanol were not sampled in May.

In the overall assessment and quality comparison of fuel samples taken in May compared to April, CTIA saw an improvement from 1.2% to 0.6% in the number of non-compliant samples.

Fuel quality inspection results by inspectorate:

| Inspectorate | Petrol |  | Diesel |  | LPG |  | CNG |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | COLL | N | COLL | N | COLL | N | COLL | N | OD | N |
| Central Bohemia and City of Prague | 9 | 0 | 12 | 0 | 6 | 0 | 1 | 0 | 28 | 0 |
| South Bohemia and Vysočina | 12 | 0 | 12 | 1 | 0 | 0 | 0 | 0 | 24 | 1 |
| Pilsen a Karlovy Vary | 12 | 0 | 12 | 0 | 6 | 0 | 1 | 0 | 31 | 0 |
| Ústí a Liberec | 12 | 0 | 12 | 0 | 0 | 0 | 0 | 0 | 24 | 0 |
| Hradec Králové a Pardubice | 12 | 0 | 12 | 0 | 6 | 0 | 1 | 0 | 31 | 0 |
| South Moravian a Zlín | 9 | 0 | 12 | 0 | 0 | 0 | 0 | 0 | 21 | 0 |
| Olomouc a Moravian-Silesian | 9 | 0 | 12 | 0 | 0 | 0 | 0 | 0 | 21 | 0 |
| Total | 75 | 0 | 84 | 1 | 18 | 0 | 3 | 0 | 180 | 1 |

Legend: COLL = collected, N = noncompliant